Project Name: Automatic Light Control

# Introduction

The Automatic light control system

The Automatic Light control system is an progressive and energy-efficient solution that automates lights based totally on particular situations, including door activity. This system removes the need for manual switching by using the usage of a sensor-based mechanism that detects the opening or closing of a door. The layout guarantees that lights are activated most effective when essential, retaining energy whilst presenting comfort. stimulated by way of realistic implementations, this venture integrates simple yet effective components like a comparator, timer IC, and output devices. The system is ideal for use in homes, workplaces, and public spaces in which lighting fixtures automation is a concern.

# Hardware used

- Door Switch (Normally Closed - NC):

  - Detects the door’s state (open or closed).

- Comparator (LM741 IC):

  - Compares the voltage from the door switch with a reference voltage to generate a HIGH or LOW output.

- 555 Timer IC:

  - Functions as a pulse generator to control the activation duration of the light or buzzer.

- Resistors:

  - R1 (10 kΩ): Pull-down resistor to stabilize the comparator input and prevent false triggering.

  - R2 (1 kΩ): Used with the 555 Timer to define the RC time constant.

  - R3 (470 Ω): Current-limiting resistor for the LED (if used as an output indicator).

- Capacitors:

  - C1 (10 µF): Smoothing capacitor for the power supply.

  - C2 (0.1 µF): Noise suppression capacitor near the ICs.

  - C3 (1 µF): Timing capacitor used in the 555 Timer to define the output pulse duration.

- 555 Timer Configuration:

  - The output pulse duration (time for which the light stays ON) is calculated using the formula:
T = 1.1 × R2 × C3
For example:

    - If R2 = 1 kΩ and C3 = 1 µF, the output pulse duration T = 1.1 × 1000 × 0.000001 = 1.1 ms.
Adjust R2 and C3 values based on the desired light activation time (e.g., longer durations require larger resistors or capacitors).

- Relay Module:

  - Switches the high-power load (light bulb or buzzer) based on the output of the 555 Timer.

- Light Bulb/Lamp:

  - Voltage Rating: 12V (or match it with the relay output voltage).

  - Power Rating: 5W or as required for your application.

  - Ensure the lamp's power matches the relay's capacity to avoid overloading.

- Buzzer (Optional):

  - Voltage Rating: 5V or 12V (depending on the system design).

- Power Supply:

- ICs and Logic Circuit: 5V DC (regulated).

- Relay and Light Bulb: 12V DC (for switching and output).

- Breadboard/PCB:

  - For mounting and connecting components.

- Connecting Wires:

  - For establishing electrical connections between components.

# objective

The main objectives of this project are:

Automated light control:

To automate the switching of lighting fixtures based totally on door activity the usage of a sensor-based totally mechanism.

Energy Conservation:

Reducing electricity intake by using ensuring lights are only ON while the door is opened and off while the door is closed.

Low-cost Implementation:

To design a device using low-cost and easily available electronic additives.

Ease of installation:

To increase a device that may be installed in any environment without complex configurations.

Scalability:

To enable future upgrades like integrating motion sensors or ambient light sensors for greater advanced control.

# Block Diagram

# Methodology

- Setup the Door Switch (NC):

  - Mount the switch at a position where it opens only when the door is fully pushed against the wall.

  - Ensure proper wiring from the switch to the comparator circuit.

- Comparator Design:

  - Use the LM741 operational amplifier.

  - Set up one input of the LM741 with a reference voltage.

  - Connect the other input to the door switch's output to detect voltage changes when the switch opens or closes.

- Timer Circuit Configuration:

  - Use the 555 Timer IC in monostable mode.

  - Connect the output of the comparator to the trigger input of the timer.

  - Configure the timer to maintain the output (light/buzzer) active for a predefined duration.

- Output Stage:

  - Connect the 555 Timer's output to the light or buzzer circuit.

  - Ensure the light or buzzer activates when the timer output goes HIGH.

- Testing:

  - Open the door fully and observe if the light/buzzer activates.

  - Ensure the light/buzzer turns off after the timer's duration.

  - Test the system when the door remains closed to confirm no false activations.

- Finalization:

  - Fine-tune the timer's duration as required.

  - Securely mount all components and ensure reliable operation under different conditions.