Assignment # 02

NAME: SABA SHAHID

ENROLLNMENT: 02-134241-048

CLASS: BS (CS) 3-A

CLO-2                                                                                                           PLO-2 (BT level- C3)

Circuit switching is a mechanism in which the communication path is dedicated. In contrast, packet switching mechanism allows the sharing of communication path among users. Keeping conventional and Next Generation Networks (NGN) in mind, as mentioned below, answer the followings:

- Conventional Communication Networks: develop a packet switching network and apply any real time data services application (for example voice) on it.  Demonstrate whether it is a good idea to use the real time data services over packet switching network? Support your answer with detail discussion, examples and limitations (if any).

ANSWER

  - Packet Switching Network Development:

  - Conceptual Design:

- Nodes and Links:

  - We'll create a simplified network with a few nodes (routers) connected by links (communication channels).

  - Each node will have routing capabilities to forward packets.

- Packet Structure:

  - Define a packet structure with:

    - Header: Source/destination addresses, sequence number, timestamp (for real-time), and payload length.

    - Payload: The actual voice data.

- Routing Algorithm:

  - Implement a basic routing algorithm like distance vector or link-state to determine the best path for packets.

- Buffering and Queuing:

  - Implement buffers at each node to store packets temporarily.

  - Use queuing mechanisms (e.g., FIFO) to manage packet flow.

- Voice Application Simulation:

  - Simulate a voice application by dividing voice data into packets.

  - Introduce delays and jitter to mimic real-world network conditions.

  - Implement a simple voice reconstruction at the receiving end.

    - Example Scenario:

Imagine a network with nodes A, B, C, and D. Node A sends voice packets to node D. The routing algorithm determines the path (e.g., A -> B -> C -> D). Packets might experience different delays at each node due to queuing.

  - Suitability of Real-Time Data Services (Voice) Over Packet Switching:

  - Arguments for Suitability:

- Efficiency:

  - Packet switching is efficient in utilizing network bandwidth. It only sends data when needed, unlike circuit switching, which reserves bandwidth even during silence.

- Flexibility:

  - Packet switching can dynamically adapt to changing network conditions. If a link fails, packets can be rerouted.

- Scalability:

  - Packet switching networks can easily scale to accommodate a large number of users.

- Integration:

  - Packet switching allows voice to be integrated with other data services (e.g., video, text) on the same network.

  - Arguments Against Suitability:

- Delay and Jitter:

  - Packet switching introduces variable delays and jitter (variations in delay), which can degrade voice quality.

  - Packets may arrive out of order, requiring reassembly.

- Packet Loss:

  - Packets can be lost due to network congestion or errors, resulting in gaps in the voice signal.

- Quality of Service (QoS):

  - Packet switching networks do not inherently guarantee QoS. Real-time services like voice require low latency and minimal jitter, which can be challenging to achieve.

  - These mechanisms help ensure that voice packets receive preferential treatment.

  - Examples:

1. Voice over IP (VoIP):

  - VoIP uses packet switching to transmit voice over the internet.

  - It has become a widely used technology for telephone calls.

  - VoIP applications use protocols like RTP (Real-time Transport Protocol) and SIP (Session Initiation Protocol) to manage voice communication.

- Video conferencing:

  - Video conferencing also uses packet switching.

  - It is even more sensitive to delay and jitter than voice calls.

  - Conclusion:

While packet switching offers efficiency and flexibility, it presents challenges for real-time services like voice due to delay, jitter, and packet loss. With proper QoS mechanisms and careful network design, it is possible to achieve acceptable voice quality over packet switching networks. The internet is based on packet switching, and real time services are performed on it every day. Therefore, it is possible, but needs careful engineering.

- Next Generation Networks, Infer/Predict whether all types of data (real time/ non real time) should be transported by using the circuit switching or packet switching networks? Support your answer with discussion and examples.

# ANSWER:

  - Arguments for Packet Switching in NGNs:

- Efficiency and Flexibility:

  - Packet switching excels in bandwidth utilization. It only transmits data when needed, eliminating wasted resources during idle periods (like silence in a voice call).

  - NGNs aim to integrate diverse services (voice, video, data) over a single infrastructure. Packet switching's flexibility allows for dynamic adaptation to varying data demands and types.

- Scalability:

  - NGNs are designed to handle massive amounts of data and users. Packet switching's inherent scalability makes it suitable for this purpose.

  - Adding new users or services is far simpler in a packet-switched environment.

- Cost-Effectiveness:

  - Consolidating all services onto a single packet-switched network reduces infrastructure costs compared to maintaining separate circuit-switched networks.

  - This consolidation simplifies network management and operations.

- Convergence:

  - NGNs are built on the concept of convergence, where different services share the same network infrastructure. Packet switching is the enabling technology for this convergence.

  - IP-based networks, which are inherently packet-switched, are the foundation of NGNs.

- Data Burstiness:

  - Most data traffic, including non-real-time (e.g., web browsing, file transfers) and even real-time (e.g., video streaming), is bursty in nature. Packet switching handles bursts of data more efficiently than circuit switching.

- QoS Mechanisms:

  - While circuit switching offers guaranteed bandwidth and latency, packet switching can achieve comparable QoS through advanced mechanisms like:

    - Prioritization (e.g., DiffServ)

    - Traffic shaping

    - Resource reservation (e.g., RSVP)

    - Jitter buffers (for real-time applications)

  - These QoS mechanisms can be tailored to meet the specific requirements of different data types.

- Arguments Against Circuit Switching in NGNs (and Why They're Less Relevant):

- Guaranteed Bandwidth and Latency:

  - Circuit switching provides dedicated bandwidth and low, predictable latency, which is ideal for real-time services.

  - However, as mentioned, packet switching with QoS can achieve similar results.

- Predictability:

  - Circuit switching offers predictable performance, which is important for applications that require consistent quality.

  - Again, with proper QoS, packet switching can provide the predictability needed for most real-time applications.

- Inefficiency:

  - Circuit switching is highly inefficient for bursty data traffic, wasting bandwidth during idle periods.

  - NGNs are designed to handle diverse traffic patterns, making packet switching the more efficient choice.

  - Examples:

- VoIP (Voice over IP):

  - Voice communication is increasingly carried over packet-switched networks (VoIP). Protocols like RTP and SIP are used to manage real-time voice traffic.

  - QoS mechanisms ensure acceptable voice quality.

- Video Streaming:

  - Video streaming services rely on packet switching.

  - Adaptive streaming techniques adjust video quality based on network conditions.

- Data Centers and Cloud Computing:

  - Data centers and cloud computing environments use packet-switched networks for all types of data traffic.

  - Virtualization and cloud technologies are optimized for packet-switched networks.

- 5G and Beyond:

  - 5G and future mobile networks are built on IP-based (packet-switched) architectures.

  - They support a wide range of services, including real-time applications like augmented reality and virtual reality.

  - Conclusion:

While circuit switching still has niche applications where guaranteed bandwidth and latency are paramount, packet switching is the dominant and preferred technology for NGNs. Its efficiency, flexibility, scalability, and ability to support diverse services make it the ideal choice for transporting all types of data.

The evolution of QoS mechanisms has allowed packet-switched networks to meet the demands of real-time applications.

Deadline: 5th April, 2025

Important Instructions:

  - Use Standard Font (Arial), Line Spacing of 1.15,

  - Proper Justified, Page Margins 1’’from all four sides.

  - Text Font Size = 12	Heading Font Size = 14

  - Late assignment submission will not be marked

  - Copied Assignment will be given ZERO and no resubmission will be accepted

  - Maximum Marks= 05