Exam: Communication Networks
Total Marks: 20
Time: 60 minutes

### Question 1: Fundamental Concepts (4 marks)

Part (a): Diagram and Explanation
Illustrate the OSI model layers with a diagram and explain the role of the Transport Layer in ensuring reliable communication. (2 marks)

Solution:

- Diagram:

Copy

[Application Layer]

[Presentation Layer]

[Session Layer]

[Transport Layer] ← Ensures end-to-end message delivery, error recovery, and flow control.

[Network Layer]

[Data Link Layer]

[Physical Layer]

- Explanation: The Transport Layer (e.g., TCP) segments data, manages sequencing, and retransmits lost packets to ensure reliable process-to-process delivery.

Part (b): Inference
Why is the OSI model considered a reference model, while TCP/IP is widely implemented? (2 marks)

Solution:

- OSI is theoretical, with strict layering, while TCP/IP is practical, optimized for scalability and real-world use (e.g., Internet).

### Question 2: Numericals (6 marks)

Part (a): TDM Calculation
Four 1-kbps connections are multiplexed using TDM with 1-bit slots. Calculate: (i) Link transmission rate (ii) Duration of a frame. (4 marks)

Solution:

- Link rate = 4 × 1 kbps = 4 kbps.

- Frame duration = 1 ms (same as input unit duration).

Part (b): PCM Sampling
A voice signal (max 4 kHz) is digitized using PCM. What is the minimum sampling rate? (2 marks)

Solution:

- Nyquist rate = 2 × 4 kHz = 8000 samples/sec.

### Question 3: Network Design (10 marks)

Part (a): Sketch
Design a LAN for a small office with 10 PCs, a printer, and a server using a star topology. Label devices and topology type. (2 marks)

Solution:

Copy

[Switch] ← Central hub

│─ PC1

│─ PC2

...

└─ Server & Printer

Part (b): Scenario
A company needs to connect two offices (LANs) 50 km apart. Propose a solution with a diagram, justifying your choice of devices/protocols. (6 marks)

Solution:

- Solution: Use a leased line or VPN over Internet with routers at both ends.

- Diagram:

Copy

[Office 1 LAN] ───Router───[WAN]───Router───[Office 2 LAN]

- Justification: Routers enable inter-network communication; leased line ensures dedicated bandwidth, while VPN offers cost-effective security.

Total: 20 marks

Key:

- C2 (Understand): Q1(a), Q1(b).

- C3 (Apply): Q2(a), Q3(a).

- C4 (Analyze): Q2(b), Q3(b).

### Question 1: Fundamental Concepts (8 marks)

Part (a): Diagram and Explanation
Illustrate the TCP/IP protocol stack alongside the OSI model. Compare the functionalities of the Network Layer in both models. (4 marks)

Solution:

- Diagram:

Copy

OSI Model            TCP/IP Stack

[Application]        [Application]

[Presentation]

[Session]

[Transport]          [Transport]

[Network]            [Internet]

[Data Link]          [Network Access]

[Physical]

- Comparison:

  - OSI Network Layer: Handles logical addressing, routing, and packet forwarding (e.g., IP).

  - TCP/IP Internet Layer: Equivalent to OSI Network Layer but focuses on IP-based routing and fragmentation.

Part (b): Inference
Why does the TCP/IP model combine the OSI Presentation and Session Layers into the Application Layer? Provide an example. (4 marks)

Solution:

- Reason: TCP/IP prioritizes practicality; applications (e.g., HTTP) handle encryption (Presentation) and session management (Session) internally.

- Example: HTTPS manages encryption (Presentation) and cookies (Session) within the Application Layer.

### Question 2: Numericals (12 marks)

Part (a): TDM Bandwidth Allocation
Six 2-Mbps digital signals are multiplexed using Synchronous TDM. Calculate:
(i) Output frame duration if each input slot carries 1 bit.
(ii) Total link bandwidth required.
Justify your calculations. (6 marks)

Solution:

- Frame duration = Input bit duration = 1 / 2 Mbps = 0.5 μs.

- Link bandwidth = 6 × 2 Mbps = 12 Mbps.

- Justification: Synchronous TDM allocates fixed slots per frame, scaling bandwidth linearly.

Part (b): PCM Bit Rate
A PCM system samples an analog signal at 16 kHz with 8-bit quantization. Calculate the bit rate and required bandwidth if Nyquist criteria are met. (6 marks)

Solution:

- Bit rate = Sampling rate × Bits per sample = 16 kHz × 8 = 128 kbps.

- Bandwidth ≥ 8 kHz (Nyquist: 2 × highest signal frequency).

### Question 3: Network Design (10 marks)

Part (a): Topology Design
Sketch a hybrid topology for a university campus with three buildings (Admin, Library, Labs) interconnected via fiber-optic backbone. Each building has a star LAN. Label critical devices (e.g., routers, switches). (4 marks)

Solution:

Copy

[Router] ────[Fiber Backbone]─────[Router]

│                      │                      │

[Admin Switch]    [Library Switch]    [Labs Switch]

│                      │                      │

(PCs, Printers)   (PCs, Servers)     (Lab PCs)

Part (b): Scenario Analysis
A hospital needs secure, real-time data exchange between ICU monitors (high priority) and admin PCs (low priority). Propose a solution with:
(i) A suitable multiplexing technique (justify).
(ii) A protocol to prioritize traffic.
Include a diagram. (6 marks)

Solution:

- Technique: Statistical TDM (optimizes bandwidth by dynamically allocating slots to active ICU devices).

- Protocol: DiffServ (QoS) in routers to prioritize ICU traffic.

- Diagram:

Copy

[ICU Monitors] ──[Switch]──[Router (QoS)]──[WAN]

[Admin PCs] ──────┘

Total: 30 marks

Key:

- C2 (Understand): Q1(a), Q1(b).

- C3 (Apply): Q2(a), Q3(a).

- C4 (Analyze): Q2(b), Q3(b).

Rationale for Time Extension:

- Q1: In-depth comparisons require analysis (8 mins).

- Q2: Complex calculations with justification (20 mins).

- Q3: Design and scenario-based reasoning (25 mins).

- Review: 7 mins.

Exam 2

### Communication Networks Exam

Time: 1 hour 30 minutes
Total Marks: 30

### Question 1: Protocol Architectures (8 marks)

Part (a)
Compare the OSI and TCP/IP models using a diagram. Highlight two key differences in their layer functionalities. (4 marks)

Solution:

- Diagram:

Copy

OSI (7 Layers)            TCP/IP (4 Layers)

---------------------     -----------------

Application               Application

Presentation

Session

Transport                 Transport

Network                   Internet

Data Link                 Network Access

Physical

- Differences:

  - OSI Strict Layering: Presentation/Session layers are separate; TCP/IP merges them into Application.

  - TCP/IP Flexibility: Network Access Layer combines OSI Data Link + Physical.

Part (b)
Why does HTTP use TCP instead of UDP? Relate your answer to protocol characteristics. (4 marks)

- Answer: HTTP requires reliability (TCP guarantees delivery via ACKs, retransmissions) and ordered data (e.g., webpage loading). UDP’s lack of error recovery is unsuitable.

### Question 2: Numerical Problems (12 marks)

Part (a)
Four 500-kbps channels are multiplexed using TDM with 2-bit interleaving. Calculate:

- Output frame duration.

- Total link bandwidth.
(6 marks)

Solution:

- Input bit duration = 1/500 kbps = 2 μs → Output slot = 2 μs / 2 = 1 μs.

- Frame duration = 2 μs (same as input unit).

- Bandwidth = 4 × 500 kbps = 2 Mbps.

Part (b)
A PCM system quantizes a 0-4 kHz voice signal with 256 levels. Determine:

- Bit rate.

- Minimum bandwidth.
(6 marks)

Solution:

- Bits/sample = log₂(256) = 8 → Bit rate = 8 × 8 kHz = 64 kbps.

- Bandwidth ≥ 4 kHz (Nyquist).

### Question 3: Network Design & Analysis (10 marks)

Part (a)
Design a WAN for a bank with three branches (HQ, Branch A, B) connected via leased lines. Sketch the topology and label devices (routers, firewalls). (4 marks)

Solution:

Copy

[HQ Router] ────Leased Line────[Branch A Router]

│

Leased Line

│

[Branch B Router]

Firewalls at each site.

Part (b)
A video-conferencing app experiences jitter. Propose solutions using:

- A suitable transport protocol.

- QoS mechanism.
Include a diagram of traffic prioritization. (6 marks)

Solution:

- Protocol: RTP over UDP (low latency, timestamps for jitter control).

- QoS: DiffServ in routers to prioritize video packets.

- Diagram:

Copy

[Video Traffic] ───[High-Priority Queue]───[Router]

[Email Traffic] ──[Low-Priority Queue]──┘

Marking Scheme:

- Q1: Diagram (2), Differences (2), HTTP/TCP rationale (4).

- Q2: TDM calculations (6), PCM logic (6).

- Q3: Topology sketch (4), Scenario solutions (6).

Learning Outcomes Addressed:

- C2 (Understand): Q1 protocol comparisons.

- C3 (Apply): Q2 numericals.

- C4 (Analyze): Q3 design/scenario.

Exam 3

Part (a): Diagram and Explanation
Illustrate the TCP/IP protocol stack alongside the OSI model. Compare the functionalities of the Network Layer in both models. (4 marks)

Solution:

  - Diagram:

Copy

OSI Model            TCP/IP Stack

[Application]        [Application]

[Presentation]

[Session]

[Transport]          [Transport]

[Network]            [Internet]

[Data Link]          [Network Access]

[Physical]

  - Comparison:

    - OSI Network Layer: Handles logical addressing, routing, and packet forwarding (e.g., IP).

    - TCP/IP Internet Layer: Equivalent to OSI Network Layer but focuses on IP-based routing and fragmentation.

Part (b): Inference
Why does the TCP/IP model combine the OSI Presentation and Session Layers into the Application Layer? Provide an example. (4 marks)

Solution:

  - Reason: TCP/IP prioritizes practicality; applications (e.g., HTTP) handle encryption (Presentation) and session management (Session) internally.

  - Example: HTTPS manages encryption (Presentation) and cookies (Session) within the Application Layer.

### Question 2: Numericals (12 marks)

Part (a): TDM Bandwidth Allocation
Six 2-Mbps digital signals are multiplexed using Synchronous TDM. Calculate:
(i) Output frame duration if each input slot carries 1 bit.
(ii) Total link bandwidth required.
Justify your calculations. (6 marks)

Solution:

  - Frame duration = Input bit duration = 1 / 2 Mbps = 0.5 μs.

  - Link bandwidth = 6 × 2 Mbps = 12 Mbps.

  - Justification: Synchronous TDM allocates fixed slots per frame, scaling bandwidth linearly.

Part (b): PCM Bit Rate
A PCM system samples an analog signal at 16 kHz with 8-bit quantization. Calculate the bit rate and required bandwidth if Nyquist criteria are met. (6 marks)

Solution:

  - Bit rate = Sampling rate × Bits per sample = 16 kHz × 8 = 128 kbps.

  - Bandwidth ≥ 8 kHz (Nyquist: 2 × highest signal frequency).

### Question 3: Network Design (10 marks)

Part (a): Topology Design
Sketch a hybrid topology for a university campus with three buildings (Admin, Library, Labs) interconnected via fiber-optic backbone. Each building has a star LAN. Label critical devices (e.g., routers, switches). (4 marks)

Solution:

Copy

[Router] ────[Fiber Backbone]─────[Router]

│                      │                      │

[Admin Switch]    [Library Switch]    [Labs Switch]

│                      │                      │

(PCs, Printers)   (PCs, Servers)     (Lab PCs)

Part (b): Scenario Analysis
A hospital needs secure, real-time data exchange between ICU monitors (high priority) and admin PCs (low priority). Propose a solution with:
(i) A suitable multiplexing technique (justify).
(ii) A protocol to prioritize traffic.
Include a diagram. (6 marks)

Solution:

  - Technique: Statistical TDM (optimizes bandwidth by dynamically allocating slots to active ICU devices).

  - Protocol: DiffServ (QoS) in routers to prioritize ICU traffic.

  - Diagram:

Copy

[ICU Monitors] ──[Switch]──[Router (QoS)]──[WAN]

[Admin PCs] ──────┘

Total: 30 marks

Key:

  - C2 (Understand): Q1(a), Q1(b).

  - C3 (Apply): Q2(a), Q3(a).

  - C4 (Analyze): Q2(b), Q3(b).

Rationale for Time Extension:

  - Q1: In-depth comparisons require analysis (8 mins).

  - Q2: Complex calculations with justification (20 mins).

  - Q3: Design and scenario-based reasoning (25 mins).